CPE301 – SPRING 2019

# Design Assignment 4, Part 2

Student Name: James Skelly

Student #: 2000945485

Student Email: skellj1@unlv.nevada.edu

Primary Github address: https://github.com/skellj1/submission_da

Directory: skellj1/submission_da

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Components used include Atmel Studio 7, stepper motor, servo motor, jumper wire, atmega328p, screwdriver, multifunction shield, iphone camera, driver module for stepper.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA4A_JS.c

* Created: 4/11/2019 10:54:25 AM

* Author : James Skelly

*/

// define clock for delay function, include headers

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

int adcVal;			// holds value of ADC

char toggle = 0;			// initialize toggle variable

int main(void)

{

DDRB = 0xFF;			// PORTB as output

DDRC = 0x00;			// PORTC as input

PORTB |= 0xFF;			// Set all of PORTB high initially

// set timer1 operation mode and prescaler

TCCR1A |= (1<<COM1A1)|(1<<COM1B1)|(1<<WGM11);	// use PWM for output compare pins

// OC1A/B, Fast PWM, Non-inverted mode

TCCR1B |= (1<<WGM13)|(1<<WGM12)|(1<<CS11);		// timer1 pre-scaler set to 8

ICR1 = 10000;						// set TOP value

PORTC |= (1<<PORTC1);	// set portc initially high to wait for button press

PCICR = (1<<PCIE1);	// set portc input capture enable bit

PCMSK1 = (1<<PCINT9);	// enable pin change interrupt

// Initialize ADC

DIDR0 = 0x1;		// disable the digital input on ADC0 pin

ADMUX = (1<<REFS0);	// set ADC reference pin to PC0 (Potentiometer)

ADCSRA |= (1<<ADEN) | (1<<ADPS2) | (1<<ADPS1) | (1<<ADPS0);  // enable ADC using system clock

ADCSRB = 0x0;		// free running mode

sei();				// globally enable interrupts

while(1)		// wait for pin change interrupt request

{

}

}

ISR(PCINT1_vect){

if(!(PINC & (1 << PINC2)))				// if button is pressed...

{

_delay_ms(200);						// debounce button pressed

while(!(PINC & (1 << PINC2)));

if(toggle == 1){

PORTB |= (1 << PORTB1);		// set PB1 high

PORTB &= ~(1 << PORTB5);		// turn on LED D1

ADCSRA |= (1 << ADSC);		// start conversion

while((ADCSRA&(1<<ADIF))==0);	// wait for conversion to finish

adcVal = ADC & 0x03FF;		// extract right 10-bits of ADC register

OCR1A = 10*adcVal;		// OCR1A value for duty cycle

}

else if(toggle == 0){

OCR1A = 0;			// set output compare value to 0

PORTB &= ~(1 << PORTB1);		// set PB1 low

PORTB |= (1 << PORTB5);		// turn of LED D1

}

toggle ^= 1;				// toggle the variable used to toggle

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

PART 1: Stepper Motor

/*

* Stepper_PART1.c

*

* Created: 4/18/2019 6:22:27 PM

* Author : James Skelly

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

// Function Prototypes

void timer_init(void);

void adc_init(void);

int main(void)

{

int period;    // integer used to set the period

while (1)

{

timer_init();      // initialize and set up timer to be used

adc_init();        // initialize and set up ADC

DDRB=0x0F;	// Port B low bits to output

OCR0A = ADC;	 // set value of ADC data registers as compare value for timer

// The following if statements each contain the same code with a different period,

// depending on the voltage read out of the potentiometer at PC0. The values that

// PORTB is set to (i.e. 0x09, 0x03, 0x06, and 0x0C) allow the motor to make steps

// and rotate at a speed specified by the potentiometer. Since all of this is inside

// the while(1) loop, the motor will keep spinning until OCR0A satisfies a different

// condition, in which case the motor will spin at a different speed.

if ((OCR0A>=0)&&(OCR0A<=50))

{

period =2 ;

PORTB = 0x09;

_delay_ms(period);

PORTB = 0x03;

_delay_ms(period);

PORTB = 0x06;

_delay_ms(period);

PORTB = 0x0c;

_delay_ms(period);

OCR0A = ADC;

}

else if ((OCR0A>50)&&(OCR0A<=120))

{

period=30;

PORTB = 0x09;

_delay_ms(period);

PORTB = 0x03;

_delay_ms(period);

PORTB = 0x06;

_delay_ms(period);

PORTB = 0x0c;

_delay_ms(period);

OCR0A = ADC;

}

else if ((OCR0A>120)&&(OCR0A<=150))

{

period=100;

PORTB = 0x09;

_delay_ms(period);

PORTB = 0x03;

_delay_ms(period);

PORTB = 0x06;

_delay_ms(period);

PORTB = 0x0c;

_delay_ms(period);

OCR0A = ADC;

}

else if ((OCR0A>150)&&(OCR0A<=200))

{

period=200;

PORTB = 0x09;

_delay_ms(period);

PORTB = 0x03;

_delay_ms(period);

PORTB = 0x06;

_delay_ms(period);

PORTB = 0x0c;

_delay_ms(period);

OCR0A = ADC;

}

else if (OCR0A>200)

{

PORTB = 0; // turn motor off when OCR0A approaches max value

OCR0A = ADC;

}

}

}

void timer_init(void)

{

// Clear OC0A on compare match, CTC mode, prescaler 256

TCCR0A |= (1<<COM0A1)|(1<<WGM01);

TCCR0B |= (1<<CS02);

}

void adc_init(void)

{

ADCSRA |= ((1<<ADEN)|(1<<ADSC)|(1<<ADPS1)|(1<<ADPS0));

ADMUX |= (1<<REFS0);

}

PART 2: Servo Motor

/*

* Servo.c

*

* Created: 4/18/2019 9:13:12 PM

* Author : James Skelly

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

// Function Prototypes

void timer_init(void);   // timer1 setup function

void adc_init(void);	 // ADC setup function

int main(void)

{

while (1)

{

timer_init();		// initialize timer1

adc_init();		// initialize ADC

ICR1=5000;		// ICR is TOP for FAST PWM mode 14; set top to 5000

DDRB |= (1<<PB1);	// PB1 output mode, because OC1A is internally tied to PB1

OCR1A=ADC;		// set ADC output value as compare register value

_delay_ms(500);	// call for delay as motor moves from adjusting pot value

}

}

// Clear OC1A & OC1B on compare match, FAST PWM mode, prescaler of 64

void timer_init(void)

{

TCCR1A |= (1<<COM1A1)|(1<<COM1B1)|(1<<WGM11);

TCCR1B |= (1<<WGM13)|(1<<WGM12)|(1<<CS11)|(1<<CS10);

}

void adc_init(void)

{

ADCSRA |= ((1<<ADEN)|(1<<ADSC)|(1<<ADPS1)|(1<<ADPS0));

ADMUX |= (1<<REFS0);

}

- SCHEMATICS

STEPPER MOTOR PART 1

SERVO MOTOR PART 2

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

STEPPER SETUP

SERVO SETUP

- VIDEO LINKS OF EACH DEMO

Stepper: https://www.youtube.com/watch?v=3mt-dW8fx8I

Servo: https://www.youtube.com/watch?v=HFwg0y40PsQ

- GITHUB LINK OF THIS DA

https://github.com/skellj1/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

James W. Skelly