CPE301 – SPRING 2019

# Design Assignment 6

Student Name: James Skelly

Student #: 2000945485

Student Email: skellj1@unlv.nevada.edu

Primary Github address: https://github.com/skellj1/submission_da

Directory: skellj1/submission_da

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Components used for this DA include breadboard, Xplained Mini, jumper wire, MPU6050 Accelerometer/Gyroscope module, FTDI Chip (USART), iPhone for recording, USB cable, Atmel Studio 7.

A block diagram is shown below describing the flow of data in this DA.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA6.c

*

* Created: 5/4/2019 8:06:22 PM

* Author : J. Skelly

*/

#define F_CPU 16000000UL		// Define CPU clock Frequency

// include necessary headers for operation of program

#include <avr/io.h>

#include <util/delay.h>

#include <inttypes.h>

#include <stdlib.h>

#include <stdio.h>

#include "MPU6050_res_define.h"

#include "I2C_MasterH.h"

#include "USART_RS232_H.h"

// Function prototyping

void scale_values(void);

void send_values(void);

// initializing variables

char buffer[20], float_[10];

float Ax,Ay,Az,t;

float Gx=0,Gy=0,Gz=0;

float Acc_x,Acc_y,Acc_z;

float Temperature;

float Gyro_x,Gyro_y,Gyro_z;

int main()

{

I2C_Init();		       // I2C init function call

MPU6050_Init();	       // Initialize MPU6050

USART_Init(9600);		// Initialize USART, BAUD RATE = 9600

while(1)

{

Read_RawValue();	// call function to read raw values

scale_values();	// call function to scale values into proper units

send_values();	// send values to USART

_delay_ms(1000);

}

}

void Read_RawValue()			// Read values from gyro, wait for acknowledgement

{

MPU_Start_Loc();

Acc_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Acc_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Acc_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Nack());

I2C_Stop();

}

void scale_values()

{

// Scale accelerometer values into G force units

Ax = Acc_x/16384.0;

Ay = Acc_y/16384.0;

Az = Acc_z/16384.0;

// Scale gyroscope values into degrees/second units

Gx = Gyro_x/16.4;

Gy = Gyro_y/16.4;

Gz = Gyro_z/16.4;

}

void send_values()

{

// Send values to UART, formatting, unit outputs

dtostrf( Ax, 3, 2, float_ );

sprintf(buffer," Acc_x = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Ay, 3, 2, float_ );

sprintf(buffer," Acc_y = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Az, 3, 2, float_ );

sprintf(buffer," Acc_z = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Gx, 3, 2, float_ );

sprintf(buffer," Gyro_x = %s%c/s\t",float_,0xF8);

USART_SendString(buffer);

dtostrf( Gy, 3, 2, float_ );

sprintf(buffer," Gyro_y = %s%c/s\t",float_,0xF8);

USART_SendString(buffer);

dtostrf( Gz, 3, 2, float_ );

sprintf(buffer," Gyro_z = %s%c/s\r\n",float_,0xF8);

USART_SendString(buffer);

}

void MPU6050_Init()			// Gyro initialization function

{

_delay_ms(150);		// Power up time >100ms

I2C_Start_Wait(0xD0);	// Start at device that will be written to address

I2C_Write(SMPLRT_DIV);	// Write to sample rate register

I2C_Write(0x07);		// set 1KHz sample rate

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(PWR_MGMT_1);	// Write to power management register

I2C_Write(0x01);		// X axis gyroscope reference frequency

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(CONFIG);		// Write to Configuration register

I2C_Write(0x00);		// Fs = 8KHz

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(GYRO_CONFIG);	// Write to Gyroscope config. register

I2C_Write(0x18);		// Full scale range +/- 2000 degree/C

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(INT_ENABLE);	// Write to interrupt enable register

I2C_Write(0x01);

I2C_Stop();

}

void MPU_Start_Loc()

{

I2C_Start_Wait(0xD0);	// I2C start with device write address

I2C_Write(ACCEL_XOUT_H);	// Write start location address from where to read

I2C_Repeated_Start(0xD1);	// I2C start with device read address

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Not applicable for this assignment.

- SCHEMATICS

The below schematic was generated using Fritzing’s Breadboard Schematic Creator.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=ROYeCyrbeLc

- GITHUB LINK OF THIS DA

https://github.com/skellj1/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

James W. Skelly