CPE301 – SPRING 2019

# MIDTERM 2

Student Name: James Skelly

Student #: 2000945485

Student Email: skellj1@unlv.nevada.edu

Primary Github address: https://github.com/skellj1/submission_da

Directory: skellj1/submission_da

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/Midterm, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Components include : Xplained Mini, ESP Module, FTDI Chip, APDS9960, Breadboard.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* Midterm 2

*

* Created: 5/8/2019 1:43:56 AM

* Author : Skellj1

*/

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdlib.h>

#include <stdint.h>

#include "SparkFun_APDS9960.h"

#include "i2c_master.h"

#define F_CPU 16000000UL

#define BAUD 9600

#define FOSC 16000000

#define UBRREQ FOSC/16/BAUD -1

#define APDS9960_WRITE 0x72

#define APDS9960_READ 0x73

void UART_init (void);

void APDS_init (void);

int uart_putchar( char c, FILE *stream);

FILE str_uart = FDEV_SETUP_STREAM(uart_putchar, NULL , _FDEV_SETUP_WRITE);

void ReadValues(void);

// Initialize variables for field readings of red, green, blue light

uint16_t RED, GREEN, BLUE;

char sred[5], sgreen[5], sblue[5];

int main( void )

{

UART_init();			// UART Initialization

APDS_init();			// Sensor Module Initialization

i2c_init();				// I2C Initialization

stdout = &str_uart;

RED = 0;				// Initialize RED read variable to 0

GREEN = 0;				// Initialize GREEN read variable to 0

BLUE = 0;				// Initialize BLUE read variable to 0

_delay_ms(2000);

printf("AT\r\n");			// AT initial communication confirmation

_delay_ms(5000);

printf("AT+CWMODE=3\r\n");	// AP + Station Mode

_delay_ms(5000);

printf("AT+CWJAP=\"[Network]\",\"[Password]\"\r\n");// WIFI information inserted here

while(1)

{

_delay_ms(5000);

printf("AT+CIPMUX=0\r\n");		// Initial connection

_delay_ms(5000);

printf("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n");// connect to cloud

_delay_ms(5000);

ReadValues();				// call function to read values from sensor

printf("AT+CIPSEND=104\r\n");

// channel key for thingspeak (API Key excluded)

printf("GET https://api.thingspeak.com/update?api_key=[API_KEY_GOES_HERE]&field1=0%05u&field2=%05u&field3=%05u\r\n", RED, GREEN, BLUE);

_delay_ms(3000);

}

}

void UART_init(void)

{

// Set baud rate

uint16_t BAUDRATE = UBRREQ;

UBRR0H = BAUDRATE >> 8;

UBRR0L = BAUDRATE & 0xFF;

// Enable receive/transmit

UCSR0B = ( 1 << RXEN0)|( 1 << TXEN0);

// 8 data bit, one stop bit

UCSR0C = (3 << UCSZ00);

}

void APDS_init(void)

{

uint8_t SETUP;

i2c_readReg(APDS9960_WRITE, APDS9960_ID, &SETUP,1);

if(SETUP != APDS9960_ID_1) while(1);

SETUP = 1 << 1 | 1<<0 | 1<<3 | 1<<4;

i2c_writeReg(APDS9960_WRITE, APDS9960_ENABLE, &SETUP, 1);

SETUP = DEFAULT_ATIME;

i2c_writeReg(APDS9960_WRITE, APDS9960_ATIME, &SETUP, 1);

SETUP = DEFAULT_WTIME;

i2c_writeReg(APDS9960_WRITE, APDS9960_WTIME, &SETUP, 1);

SETUP = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS9960_WRITE, APDS9960_PPULSE, &SETUP, 1);

SETUP = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS9960_WRITE, APDS9960_POFFSET_UR, &SETUP, 1);

SETUP = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS9960_WRITE, APDS9960_POFFSET_DL, &SETUP, 1);

SETUP = DEFAULT_CONFIG1;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG1, &SETUP, 1);

SETUP = DEFAULT_PERS;

i2c_writeReg(APDS9960_WRITE, APDS9960_PERS, &SETUP, 1);

SETUP = DEFAULT_CONFIG2;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG2, &SETUP, 1);

SETUP = DEFAULT_CONFIG3;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG3, &SETUP, 1);

}

int uart_putchar(char c, FILE *stream)

{

//	wait here

while ( !( UCSR0A & ( 1 <<UDRE0)) );

// insert data into buffer

UDR0 = c;

return 0;

}

void USART_putstring(char *StringPtr)

{

while ((*StringPtr != '\0'))

{

while (!(UCSR0A & (1 << UDRE0))); // loop until UDRE0 is high

UDR0 = *StringPtr;		     // grab value from pointer address

StringPtr++;			// Increment string pointer character by character

}

}

// Function to read values from sensor

void ReadValues()

{

uint8_t redHigh, redLow;

uint8_t greenHigh, greenLow;

uint8_t blueHigh, blueLow;

i2c_readReg(APDS9960_WRITE, APDS9960_RDATAH, &redHigh, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_RDATAL, &redLow, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_GDATAH, &greenHigh, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_GDATAL, &greenLow, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_BDATAH, &blueHigh, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_BDATAL, &blueLow, 1);

// bit manipulation to format values read in from sensor

RED = (redHigh << 8) | redLow;

GREEN = (greenHigh << 8) | greenLow;

BLUE = (blueHigh << 8) | blueLow;

// If statements to cap value at 255 to keep an 8-bit value

if (RED > 255)

{

RED = 255;

}

if (GREEN > 255)

{

GREEN = 255;

}

if (BLUE > 255)

{

BLUE = 255;

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Not applicable for this assignment.

- SCHEMATICS

The schematic above was drafted using fritzing. The shield is shown here simply for access to the Xplained Mini pins, which correspond to the pins on the shield. The ESP module was wired up, powered with the on board 3.3V power supply, grounded to the board, and its transmit and receive pins were connected to the receive and transmit pins of the mini.

The FTDI module was also connected by Tx/Rx to both the WiFi module and the mini.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Outputting Values from Sensor in ESPlorer

Outputting Values from Sensor in Data Visualizer in Atmel Studio

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Encountered Problems

Unfortunately, I was not able to get ThingSpeak properly working for this assignment. I used the same ESP Module that I used for midterm 1, and my ESP Module was working just fine for that midterm. However, for some reason, even when connected to the wifi, and even with the channel key entered, the light on my ESP module would still flash indicating that receiving and transmitting of data was occurring. Regardless the data never showed up in ThingSpeak.

Being finals week, time constraints were tough, and I was unable to get ThingSpeak working. If I had been able to get it working, my plan was, as is seen above, to have three separate fields all in the same channel that would output the RED, GREEN, and BLUE values as a function of time, with the Y-axis ranging from 0 to 255. If I had more time, I would have gotten a new ESP Module and tried to flash the firmware onto that newer module and reattempt to upload data to ThingSpeak.

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=yr3-6CnEsH4

- GITHUB LINK OF THIS DA

https://github.com/skellj1/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

James W. Skelly